SECTION 122413 - ROLLER WINDOW SHADES

creativewindowscommercial.com

GUIDE SPECIFICATION: EDIT TO SUIT PROJECT PER BLUE BOX REQUIREMENTS.

DELETE BOXES AFTER REVISIONS ARE MADE.

- - GENERAL

  - RELATED DOCUMENTS

    - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

  - SUMMARY

    - Section Includes:

Delete items 1 through 10 below that are not required for project.

      - Manually operated sunscreen roller shades.

      - Manually operated room-darkening shades.

      - Manually operated double-roller sunscreen and room-darkening shades.

      - Top mount stationary panels

      - Electrically operated sunscreen roller shades.

      - Electrically operated room-darkening shades.

      - Electrically operated double-roller sunscreen and room-darkening shades.

      - Zipscreen manual roller window shades.

      - Zipscreen motorized roller window shades.

      - Skylight motorized roller window shades.

  - RELATED REQUIREMENTS

Delete items 1 through 4 below that are not required for project.

      - Section [061000 "Rough Carpentry"] [061053 "Miscellaneous Rough Carpentry"] for wood blocking and grounds for mounting roller shades and accessories.

      - Section 079200 "Joint Sealants" for sealing the perimeters of installation accessories for light-blocking shades with a sealant.

      - Section 092900 "Gypsum Board" for coordination with installation of shade pockets, closure, and related accessories.

      - Section 16 - Electrical: Electric service for motor controls.

      - REFERENCES

    - ASTM G 21 - Standard Practice for Determining Resistance of Synthetic Polymeric Materials to Fungi.

    - NFPA 70 - National Electrical Code.

    - NFPA 701-99 - Fire Tests for Flame-Resistant Textiles and Films

    - ACTION SUBMITTALS

    - Product Data: For each type of product.

      - Preparation instructions and recommendations.

      - Styles, material descriptions, individual components, dimensions, performance criteria, profiles, features, finishes and operating instructions.

      - Storage and handling requirements and recommendations.

      - Installation and maintenance instructions.

    - Shop Drawings: Show window openings, window treatment type, components, dimensions, clearance requirements, blocking requirements, and attachment method.

      - Field Measurements: Verify actual measurements for openings before fabrication, and after finish work is complete including painting and window installation. Indicate verified field measurements on the Shop Drawings.

      - Provide installation templates for Work installed by others.

    - Window Shade Schedule: Submit room list, field verified window dimensions, quantities, shade type, fabric, and color.

    - Samples: Fabrics and metal finishes for initial and final selections by Architect.

      - Selection Samples:  For each finish product specified, two (2) samples representing manufacturer's full range of available colors, materials and patterns.

      - Verification Samples:  For each finish product specified, two (2) full-size samples, representing actual product, color, material and pattern.

Delete F below if project is not intended for LEED certification.

    - Sustainable Design (LEED) Submittals: Submit the following in accordance with the requirements of SECTION 018113, LEED REQUIREMENTS:

      - LEED Credit MR, Materials & Resources. Submit completed LEED-NC 2.2 Submittal Templates in accordance with Section 017419, CONSTRUCTION WASTE MANAGEMENT, and other required paperwork as follows:

Select MR 5.1 or MR 5.2 or delete both if project site is not within 500 miles of fabrication plant.

        - MR 5.1: Regional Materials, Regionally Extracted, Processed & Manufactured Products, 10 Percent

        - MR 5.2: Regional Materials, Regionally Extracted, Processed & Manufactured Products, 20 Percent

          - Submit product data indicating name of the manufacturer, product cost, distance between the project site and the manufacturer, and the distance between the project site and the extraction site for each raw material

      - LEED Credit EQ, Indoor Environmental Quality. Submit completed LEED-NC 2.2 Submittal Templates and required paperwork as follows:

        - EQ 8.1, Daylight and Views: Submit proof of daylight redirection and/or glare control devices; exterior and interior permanent shading devices.

        - EQ 8.2, Daylight and Views: Submit proof of interior shading devices.

    - Quality Assurance Submittals per SECTION 014000

      - Test and Evaluation Reports: Submit certified test results by a recognized testing laboratory in accordance with specified test methods for each product and/or system indicating physical, chemical and performance characteristics.

      - Qualification Statements: Submit a letter, on printed letterhead and signed by an officer of the firm, for each listed quality assurance qualification listed, attesting to meeting each requirement called out.

- CLOSEOUT SUBMITTALS

    - Closeout Submittals per SECTION 017000 or 017800, and as follows:

      - Submit Maintenance Contract Proposals

      - Submit Operation & Maintenance Manuals

      - Submit Executed Warranty Documentation.

      - Submit Record Documents

      - Submit Sustainable Design Closeout Documentation including completed USGBC LEED® Templates for the following credits:

- QUALITY ASSURANCE

    - Qualifications per SECTION 014000, and as follows:

      - Manufacturer Qualifications: Firms experienced a minimum ten (10) years in manufacturing products or systems similar to those indicated for this Project and with a record of successful in-service performance.

        - Manufacturer, subsidiary or licensed agent shall be approved to supply the products specified and to honor any claims against product presented in accordance with warranty.

      - Fabricator / Installer Qualifications: A firm certified by manufacturer and experienced a minimum ten (10) years in producing products similar to those indicated for this Project and with a record of successful in-service performance, as well as sufficient production capacity to produce required units.

        - Installation performed with skilled, experienced and trained workmen supervised by trained personnel who shall have a minimum three (3) years successful experience in installations of similar size and scope.

    - Certifications:  Provide textile mill qualifications indicating shade cloths manufactured by a company with current ISO-9001 Certification.

    - Benchmark Installations: Provide mockup installations to verify final selections made under sample submittals, to demonstrate aesthetic effects and set quality standard benchmark installations for materials and execution.

      - Install mockup installations for benchmarking of each window treatment type in the Work as indicated by the Drawings or as directed by the Architect.

      - Approved mockups shall be benchmark installations and may be incorporated into the Work if accepted without exception by the Architect.

- DELIVERY, STORAGE, AND HANDLING

    - General: Strictly follow manufacturer’s recommendations, Section 016000, Product Requirements, and as follows:

      - Delivery and Acceptance Requirements

        - Do not deliver window shades until building is enclosed and construction within spaces where shades will be installed is substantially complete.

        - Deliver products in manufacturer's original, unopened, clean, dry and undamaged containers with labels intact.

        - Label containers and shades according to Window Shade Schedule and approved Shop Drawings.

      - Storage and Handling Requirements

        - Store and dispose of solvent-based materials, and materials to be used with solvent-based materials, in accordance with requirements of local AHJ.

      - Packaging Waste Management

        - Request that manufacturers, fabricators, suppliers and shippers shall provide least amount of packaging that adequately and properly protects, supports and contains the items shipped, and is reusable, returnable or recyclable.

- WARRANTY

    - Manufacturer’s Warranty per SECTION 016000, and as follows:

      - Hardware and Shade Fabric: Provide ten (10) year limited warranty.

      - Motors and Controls: Provide five (5) year limited warranty.

      - Roller Shade Components:  Manufacturer's limited lifetime warranty against defects in materials and workmanship.

      - Roller Shade Installation: One year from date of Substantial Completion, not including scaffolding, lifts or other means to reach inaccessible areas.

- -  PRODUCTS

- MANUFACTURERS

    - Acceptable Manufacturer: OpenLight Roller Shade Systems by Creative Windows., 2216 S. Industrial Highway, Ann Arbor, MI  48104; 734.769.5100; Fax: 734.769.5614; www.openlightshades.com

Choose B, C, or D below, deleting the ones not used.

    - Substitutions:  Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01600.

    - Alternates: The following products and manufacturers may be bid as an alternate product in accordance with Section 01030. Any pricing for alternate products shall be listed separately from the base bid specified product.  Any alternate pricing must include line-by-line compliance or non-compliance with the specifications. If the alternate product is acceptable to the Architect, the specified manufacturer will be given the opportunity to provide an equivalent proposal.

      - OpenLight Roller Shade Systems by Creative Windows.

      - (List other manufacturer or product here.)

- APPLICATIONS/SCOPE

Delete items 1 through 6 below that are not required for project.

Coordinate shade types with designations on drawings.

    - Roller Shade Schedule:

      - Shade Type 1: Manual operating, chain drive, sunscreen roller shades in all exterior windows of rooms and spaces shown on the Drawings.

      - Shade Type 2: Manual operating interior, chain drive room darkening roller shades with blackout fabric in all exterior windows of rooms and spaces shown on Drawings, and related mounting systems and accessories.

      - Shade Type 3: Manual operating interior, chain drive "double" solar and room darkening blackout roller shades, operating independently of each other, in all exterior windows of rooms and spaces shown on Drawings, and related mounting systems and accessories.

      - Shade Type 4: Motorized interior solar roller shades in all exterior windows of rooms and spaces shown on Drawings, and related motor control systems.

      - Shade Type 5: Motorized interior room darkening roller shades with blackout fabric in all exterior windows of rooms and spaces shown on Drawings, and related motor control systems.

      - Shade Type 6: Motorized interior “double”, solar and room darkening blackout roller shades, operating independently of each other, in all exterior windows of rooms and spaces shown on Drawings, and related motor control systems.

- SHADE CLOTH

    - Light-Filtering Fabric: Woven fabric, stain and fade resistant.

      - Source: Texstyle, Rollease Acmeda

      - Type: 3000 NET, PVC-coated polyester

      - Weave: Basketweave (mesh)

      - Thickness: 0.022in (.55mm)

      - Weight: 12.39oz/yd2 (420g/m2)

      - Orientation on Shadeband: Up the bolt

      - Openness Factor: 5 percent.

      - Color: As selected by Architect from manufacturer's full range

Delete B below if only 1 shade fabric is required.

    - Light-Filtering Fabric: Woven fabric, stain and fade resistant.

      - Source: [Roller shade manufacturer] <Insert source for custom fabrics>.

      - Type: [PVC-coated polyester] [Woven polyester and PVC-coated polyester] [PVC free polyester] <Insert description>.

      - Weave: [Basketweave (mesh)] [Twill pattern, two-sided fabric] <Insert description>.

      - Thickness: <Insert thickness>.

      - Weight: <Insert oz./sq. yd. (g/sq. m)>.

      - Orientation on Shadeband: [Up the bolt] [Railroaded] [As indicated on Drawings] <Insert requirements>.

      - Openness Factor: [1] [3] [5] [10]  <Insert number> percent.

      - Color: [As indicated on Drawings] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

Delete C below if no blackout fabric is required.

    - Light-Blocking Fabric: Opaque fabric, stain and fade resistant.

      - Source: [Roller shade manufacturer] <Insert source for custom fabrics>.

      - Type: [Polyester with foamed-acrylic backing] [Acrylic coated fiberglass] <Insert description>.

      - Thickness: <Insert thickness>.

      - Weight: <Insert oz./sq. yd. (g/sq. m)>.

      - Orientation on Shadeband: [Up the bolt] [Railroaded] [As indicated on Drawings] <Insert requirements>.

      - Features: [Fire-retardant] [Washable] [Antistatic treatment] <Insert requirements>.

      - Color: [As indicated on Drawings] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

Delete 2.4 below if stationary panels are not required.

- TOP MOUNT STATIONARY PANEL

    - Window Shade Panels: Fixed, rigid aluminum framed panels of fabric installed on the [interior] [exterior] of [windows] [sloped glazing] [skylights]. Assembly to mounting hardware and other components necessary for complete installation.

    - Framing: Extruded aluminum perimeter framing.  Dimensions, wall thickness, and material selected by manufacturer to accommodate panel size and type of installation.  Minimum frame size of 3/4 by 7/16 inch (19 by 11 mm). [Provide center cross braces for panels exceeding 40 inches]

      - Attachment Clips:  Wing and screws of sizes recommended by manufacturer for type of installation.  Clips designed for easy removal of panels for maintenance and cleaning.  Clips and screw heads to match finish of frame.

Delete 2.5 and 2.6 below if ONLY stationary panels are used.

- SHADE BAND

    - Shade Bands:  Construction of shade band includes the fabric, the hem weight, hem-pocket, shade roller tube, and the attachment of the shade band to the roller tube.  Sewn hems and open hem pockets are not acceptable.

      - Hem Pockets and Hem Weights:  Fabric hem pocket with welded seams (including welded ends) and concealed hem weights.  Hem weights shall be of appropriate size and weight for shade band.  Hem weight shall be continuous inside a sealed hem pocket.  Hem pocket construction and hem weights shall be similar, for all shades within one room.

      - Shade band and Shade Roller Attachment:

        - Use extruded aluminum shade roller tube of a diameter and wall thickness required to support shade fabric without excessive deflection.  Roller tubes less than 1.55 inch (39.37 mm) in diameter for manual shades are not acceptable.

        - Provide for positive mechanical engagement with drive / brake mechanism.

        - Provide for positive mechanical attachment of shade band to roller tube

- SHADE FABRICATION

    - Fabricate units to fill existing openings in their entirety, from top to bottom and from side to side, with proper deductions to protect walls and sills from abrasion and to protect shade cloth from abrasion, unless specifically indicated otherwise.

    - Fabricate shade cloth to hang flat without buckling, distortion, waves, cupping, or warps. Shade edges shall not fray, curl or unravel.  Fabricate hem as follows:

Choose 1 or 2 below, deleting the one not used.

      - Standard Bottom Bar: Concealed heat seamed fabric, rolled over an aluminum bar.

      - Wrapped Bottom Bar Option: Extruded aluminum bar face wrapped with selected shade cloth material and attached with spline insert.

    - Provide battens in standard shades to assure proper tracking and uniform rolling of the shade bands.

      - Contractor shall be responsible for assuring the width-to-height (W:H) ratios shall not exceed manufacturer's standards or, in absence of such standards, shall be responsible for establishing appropriate standards to assure proper tracking and rolling of the shade cloth within specified standards.

      - Battens shall be roll-formed stainless steel or tempered steel, as required.

    - For railroaded shade bands, provide seams in railroaded multi-width shade bands as required to meet size requirements and in accordance with seam alignment as acceptable to Architect.

      - Seams shall be properly located.

      - Furnish battens in place of plain seams when the width, height, or weight of the shade exceeds manufacturer's standards.

      - In absence of such standards, assure proper use of seams or battens as required to, and assure the proper tracking of the railroaded multi-width shade bands.

Delete 2.7 below if only manual shades are used on project.

- MOTOR FABRICATION

    - Tube Preparation

      - Cut tube to required length, to fit installation width, motor end clearance, and idler end clearance.

      - Remove each burr from tube ends and ensure tube interior is clean.

    - Tubular Motor Preparation

      - Fit drive wheel onto motor output shaft.

      - Secure drive wheel to tube using four 7/32 inch (5.56mm) steel pop rivets or four 1/4 inch (6.35mm) dia. screws.  Use only fasteners with steel grades SAE 5 or higher.  Metric fasteners shall be grade 8.8 or higher.

      - Fit end plug into other side of tube and secure with three steel pop rivets.

    - Supply Cable Output

      - Motor comes equipped with permanent 110 volt power whip w/ standard 3 prong UL approved plug and either 3 wire 18 gauge dry contact communication cable or 6 pin internet protocol communication cable

- COMPONENTS

    - Access and Material Requirements:

      - Provide shade hardware allowing for the removal of shade roller tube from brackets without removing hardware from opening and without requiring end or center supports to be removed.

      - Provide shade hardware that allows for removal and re-mounting of the shade bands without having to remove the shade tube, drive or operating support brackets.

      - Provide all shade installation hardware allowing for installation and removal of the shade without requiring the removal of hardware.

        - Brackets to be constructed of minimum 0.07 inch thick painted or nickel-plated, C1008/1010 cold rolled steel.

        - Painted brackets shall be finished with high quality baked enamel coating.

        - End plug bracket shall have a spring locking retainer device.

        - Brackets shall be reversible for right- or left-hand installation.

Delete B below if motorized shades are not used on project.

    - Motorized Shade Hardware and Shade Brackets: Fabricate using field dimensions as noted on approved Shop Drawings.

#### Brackets: Zinc plated stamped steel with suitable for ceiling, wall or jamb mounting. Provide size compatible with roller and motor.

      - Motor Angle Bracket: Provides idler end support.

        - Max. Torque: 40 Nm

        - Compatible with Universal Motor Bracket

      - Universal Motor Bracket:

        - Max. Torque: 25 Nm

        - Compatible with Motor Angle Bracket

        - Maximum Load: 126 lbs (57 kg)

Delete C below if manual shades are not used on project.

    - Manual Operated Chain Drive Hardware and Brackets:

      - Provide for universal, regular and offset drive capacity, allowing drive chain to fall at front, rear or non-offset for all shade drive end brackets.  Universal offset shall be adjustable for future change.

      - Provide hardware capable for installation of a removable fascia, for both regular and/or reverse roll, which shall be installed without exposed fastening devices of any kind.

      - Provide positive mechanical engagement of drive mechanism to shade roller tube. Friction fit connectors for drive mechanism connection to shade roller tube are not acceptable.

      - Provide control system consisting of Rollease Acmeda Skyline brand clutch combined with a chain tensioner operating system.

      - Control system shall be constructed of fiberglass reinforced polyester thermos-polymer (PBT) resin, aluminum.

      - Internal springs shall be constructed of tempered high carbon steel for smooth, trouble-free operation, precise control.

      - Clutch shall develop no more than 1/2 pound drag for easy lift.

      - System to be symmetrical for left or right hand installation.

      - Drive Chain: #10 qualified stainless steel chain rated to 90 lb. (41 kg) minimum breaking strength.

Delete D below if Zipscreen shades or skylight shades are not used on project.

Modify text in D to suit project.

    - [Zipscreen Roller Shades] and [Skylight Roller Window Shades]

Delete 1 below if Zipscreen manual shades are not used on project.

      - Control System: Tempered steel helical torsion spring of size and capacity to accomplish the lift requirements of size and fabric weight.  A speed reducer shall be an integral component of the spring assembly, with fittings sized for roller tube inside diameter and to be secured by the spring bracket end plugs. Standard 78mm overall tube diameter. Manual lowering and auto-raising of shades with adjustable top and bottom stop positions, or hand crank (up to 20-pound shade weight).

      - Mounting Hardware: Manufacturer’s standard powder-coated, cold-rolled steel brackets designed for easy installation and removal of shade unit.

      - Roller Tube: Extruded aluminum or Roll-formed steel shade roller tube of 78mm diameter to support shade fabric without excessive deflection, with engineered wall & ribs to lock the spring mechanism and stop mechanism into place, providing strength & durability.

      - Fabric Attachment to Tube: Provide for positive mechanical attachment of fabric to roller tube via Spline attachment - shall consist of a PVC spline heat-welded to the shade fabric and inserted into a channel on the roller tube.

      - Fabric Attachment to Side Channels: Provide for positive mechanical attachment of fabric to side channels via patented zip system - shall consist of a number 6 zip heat-welded to the shade fabric and inserted into a channel on the floating side channel.

      - Hem Bar: Exposed aluminum extruded hem bar, of manufacturer’s standard configuration, with coordinating end caps.  Color / finish as selected by Architect from manufacturer’s full range. D30 profile (standard) or HD hem bar (optional) both have bottom extrusion feature to add light blocking brush or bubble seal for blocking light against the sill.

      - Tension system: Tension is maintained in the fabric via positive attachment to bottom tube via 6mm lift tape or 1.5mm abrasion resistant cord.  Tension is provided by a patented modular spring mechanism for constant tension.

Choose either 8 or 9 below for side guide selection, deleting the one not used.

      - Zipscreen Side Channels: Manufacturer’s patented two-piece side channel system, made from extruded aluminum with powder-coated or anodized finish to resist exterior elements, and capable of retaining a 16’ wide shade with solid fabric in up to 23 mph wind, or a 12’ wide shade in up to 30 mph wind. Fabric guides and bottom caps for fabric protection and finished look.

      - Wire Guide Support Option:  Top and bottom anchors for 3.2mm stainless-steel wire guide cables. Must have tension adjustment and integral wire guides in hem bar end caps. Must be zinc plated or stainless brackets to resist corrosion.

Delete 2.9 below if single or dual motorized shades, Zipscreen motorized shades, or skylight motorized shades are not used on project.

Choose A, B, C, and/or D below, deleting the ones not used.

Edit blue text in each section to suit project.

- SHADE MOTOR AND CONTROL SYSTEM

    - Standard Motors with Wired Controls: Provide factory-assembled, shade-operator system, complete with electric motor and factory-prewired motor controls with a connector that disconnects motor from power, enclosures protecting controls and operating parts, and accessories necessary for a complete installation. Include wiring from motor controls to motors. Coordinate operator wiring requirements and electrical characteristics with building electrical system.

      - Electrical Components: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

        - Basis-of-Design Electric Motor: Stealth Motor; tubular, enclosed in roller.

        - Motor Electrical Characteristics: [120-V ac, 60 Hz] [24-V dc].

          - Power Cable: [Three-wire cable with 3 prong grounded plug (for 120-V ac motor)] [Three-wire cable with quick disconnect plug (with 120-V ac motor)] [Two-conductor hard-wired cable; 7.5 ft. (2.29 m) long (for 24-V dc].

        - Shade Characteristics of Motor:

          - Maximum Weight Capacity: Motor range to provide 17.7 to 443 in-lbf (2 to 50 Nm) lifting capacity options. [As required to operate roller shades indicated]

        - Single Motor Control:

          - Wired Individual Button Control Station: Momentary-contact, wall-switch-operated control station with open, close, and center off functions.

            - Keyed Control Station: Keyed, [maintained] [momentary]-contact, three-position, switch-operated control station with open, close, and off functions. Provide two keys per station.

            - Switch Control Station: [Maintained] [Momentary]-contact, wall-switch-operated control station with open, close, and center off functions.

            - Switch Positions: [Three] [Five].

            - Switch Style: [Toggle] [Rocker].

        - Group Motor Control:

          - Wired Group Control Station: Managed control of individual or groups of standard motors though a microcontroller-based device.

            - Individual Control: One motor at a time.

            - Group Control: Definable group.

            - User Interface: 6-button keypad.

      - Microprocessor Control: Electronic programmable means for setting, changing, and adjusting control features; isolated from voltage spikes and surges.

        - Basis-of-Design Product: Somfy Systems; IGC4N1.

      - Limit Switches: Adjustable switches, interlocked with motor controls and set to stop shade movement automatically at fully raised and fully lowered positions.

      - Operating Features:

        - Time calculated positioning:

          - Alignment of multiple shades in a group.

          - Three automatically set and adjustable recallable intermediate positions in between the upper and lower travel of the shade.

        - Group switching with integrated switch control; single faceplate for multiple switch cutouts.

        - Capable of interface with [audiovisual] [multiroom] [building management system] <Insert description> control system.

        - Capable of accepting input from building automation control system.

        - Override switch.

    - Radio Controlled Motors: Provide factory-assembled, shade-operator system of size and capacity and with features, characteristics, and accessories suitable for conditions indicated, complete with electric motor, enclosures protecting controls and operating parts, and accessories required for reliable operation without malfunction. Include wiring from motor controls to motors. Coordinate operator wiring requirements, radio control requirements and electrical characteristics with building electrical system.

      - Electrical Components: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

        - Basis-of-Design Electric Motor: Stealth Motor RF; tubular, enclosed in roller.

        - Motor Electrical Characteristics: [120-V ac] [24-V dc]

          - Power Cable: [Three-wire cable with 3 prong grounded plug (for 120-V ac motor)] [Three-wire cable with quick disconnect plug (with 120-V ac motor)] [Two-conductor hard-wired cable; 7.5 ft. (2.29 m) long (for 24-V dc]

        - Shade Characteristics of Motor:

          - Maximum Weight Capacity: Motor range to provide 17.7 to 443 in-lbf (2 to 50 Nm) lifting capacity options. [As required to operate roller shades indicated]

      - Radio Remote Control: Electric controls with NEMA ICS 6, Type 1 enclosure for [surface] [recessed or flush] [hand-held] <Insert type> mounting. Provide the following for remote-control activation of shades:

        - Radio / WiFi Control Interface: 433.42 MHz / 2.4GHz network with WEP, WPA2, open and mixed mode encryption; single-zone unit may be joined to create a multi-zone system; capable of serving as an Internet protocol (IP) to radio bridge for third-party control systems.

          - Basis-of-Design Product: Somfy Systems; myLink™.

          - Enclosure: ABS; UL listed; UL 94V-0 flame rating; RoHS compliant.

          - Electrical Characteristics: 110V-220V ac; 50-60 Hz; plugs into AC outlet.

          - Control Interface: [iPhone and iPad version 6 and higher] [Android and Android Tablet version Jelly Bean and higher].

          - Controllable over open application program interface (API).

          - Control Functions: [Open, Close, My] [Timer] [enable/disable sensor] [Scene].

          - Timer Control: Clock timer based on astronomic timeclock with 60-minute offset, [24-hour] [seven-day] <Insert period> programmable for regular events.

        - Hand-Held Radio Control Remote: [LCD Display] [Scroll wheel] [Tactile button]; 433.32 MHz.

          - Basis-of-Design Product: Somfy Systems; [Telis 16] [Telis 1 Chronis] [Telis] [Telis Modulis].

          - Electrical Characteristics: Battery; [2-AAA (LR3) lithium] [CR 2430 Lithium 3V].

            - Do not use rechargeable batteries.

          - Transmitter: [Sixteen-][Five-][Single-]channel.

          - Open Air Range: 65 ft. (20 m) radius through two concrete walls 11.81-inch- (30-cm-) thick or less, or 100 ft. (30.48 m) through open-air.

          - Index Protection Rating: IP 30.

          - Control Functions: [Open, Close, My] [Timer] [enable/disable sensor].

          - Timer Control: Clock timer based on astronomic timeclock with 60-minute offset, [24-hour] [seven-day] <Insert period> programmable for regular events.

          - Accessories: [Wall mount holder with screw cover and screw kit] [Decora low-voltage mud ring with faceplate] [Table top decora enclosure].

        - Radio Control [Wall Switch][ and ][Table Top]: Button-operated station.

          - Basis-of-Design Product: Somfy Systems; Decoflex Wirefree™ RTS [Table Top].

          - [Five-][Four-][Three-][Two-][Single-]channel wireless transmitter.

          - Electrical Characteristics: Battery; CR 2450 Lithium 3V.

          - Open Air Range: 65 ft. (20 m) radius through two concrete walls 11.81-inch- (30-cm-) thick or less, or 100 ft. (30.48 m) through open-air.

          - Control Functions: Open, Close, My.

          - FCC Approval: Part 15, Class B.

        - Radio Rain and Sun Sensor Control: Adjustable system consisting of digital displays detecting sun and rain intensity, responding by automatically adjusting shades. Wireless, solar-powered, LED indicator lights, optical rain-sensor and sun-level sensor with user-selected thresholds that automatically operate networked motors.

          - Basis-of-Design Product: Somfy Systems; Ondeis™ WireFree.

          - Housing material: ABS.

          - Operation Modes: 6; Demo Testing.

          - Open Air Range: 65 ft. (20 m) radius through two concrete walls 11.81-inch- (30-cm-) thick or less, or 100 ft. (30.48 m) through open-air.

          - Index Protection Rating: IP 55.

          - FCC Approved.

          - Accessories: Gutter mount bracket and socket head screws.

        - Microprocessor Control: Electronic programmable means for setting, changing, and adjusting control features; isolated from voltage spikes and surges.

          - Basis-of-Design Product: Somfy Systems; myLink™.

      - Limit Switches: Adjustable switches interlocked with motor controls and set to stop shades automatically at fully raised and fully lowered positions.

      - Operating Features:

        - Capable of interface with [audiovisual] [multiroom] [building management system] <Insert description> control system.

        - Capable of accepting input from networked building automation control system.

        - Override switch.

    - Radio Controlled Motors: Provide factory-assembled, shade-operator system of size and capacity and with features, characteristics, and accessories suitable for conditions indicated, complete with electric motor, enclosures protecting controls and operating parts, and accessories required for reliable operation without malfunction. Include wiring from motor controls to motors. Coordinate operator wiring requirements, radio control requirements and electrical characteristics with building electrical system.

      - Electrical Components: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

      - Basis-of-Design Electric Motor: Stealth Motor RF Battery; tubular, enclosed in roller.

        - Motor Electrical Characteristics: 12V DC quiet motor with built-in re-chargeable lithium ion battery and radio receiver.

        - Shade Characteristics of Motor:

          - Maximum Weight Capacity: Motor range to provide 17.7 to 177 in-lbf (2 to 20 Nm) lifting capacity options. [As required to operate roller shades indicated]

          - Motor Operating sound level <44dB

      - Charging Options:

        - Plug in 12v Charger: [1 per motor] [1 for every 10 motors] [1 per tenant space] [1 per room].

          - Plug in charger fully charges motor in 6 hours.

          - Motor operates >300 cycles on fully charged battery

          - Charger may be permanently installed, allowing battery to serve as an emergency backup feature in a power outage.

        - Solar Panel Trickle Charger: [1 per motor] [2 per motor (Y-Harness required)].

      - Control Options:

Choose any combination of control options required, deleting the ones not used.

        - Single channel wireless wall switch for radio motor control -White.

        - Two channel wireless wall switch for radio motor control -White.

        - 15 channel wireless LCD wall switch for radio motor control -White.

        - Single channel wireless handheld transmitter with concealed magnetic wall mounting clip -White.

        - 15-Channel wireless handheld LCD transmitter with concealed magnetic wall mounting clip and “Levelling Control” feature, allowing shade(s) to be operated to 10 aligned positions -White.

        - Single Channel Dry Contact Interface – Allows for full control of radio motor shade via dry contact interface. (external 12v or 24v power source required).

        - Wire-less Sun Sensor – Allows operation of one group of radio motors per sensor. 4 sensitivity level settings.

        - Wi-Fi / Serial Automation Interface.

          - Allows for full control, with feedback, of radio motor shade via RS485 serial data connection.

          - Supports Mobile Device Control through iOS or Android APPs via Wi-Fi connection.

          - Provides position reporting and “move to” position control via 2-way radio commands when operating from Mobile Device APP or connected to Automation systems.

      - Limit Switches: Adjustable switches interlocked with motor controls and set to stop shades automatically at fully raised and fully lowered positions.

      - Operating Features:

        - Capable of interface with [audiovisual] [multiroom] [building management system] <Insert description> control system.

        - Capable of accepting input from networked building automation control system.

        - Override switch.

    - Networked Automation Controlled Motors: Provide factory-assembled, shade-operator system of size and capacity and with features, characteristics, and accessories suitable for conditions indicated, complete with electric motor, enclosures protecting controls and operating parts, and accessories required for reliable operation without malfunction. Include wiring from motor controls to motors or radio controls to motors. Coordinate operator [wiring requirements] [radio controls] and electrical characteristics with building electrical system.

      - Electrical Components: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

      - Basis-of-Design Electric Motor: Stealth Motor RS485; tubular, enclosed in roller.

        - Motor Electrical Characteristics: [120-V ac] [24-V dc].

          - Power Cable: [Three-wire cable with 3 prong grounded plug (for 120-V ac motor)] [Three-wire cable with quick disconnect plug (with 120-V ac motor)] [Two-conductor hard-wired cable; 7.5 ft. (2.29 m) long (for 24-V dc].

        - Shade Characteristics of Motor:

          - Maximum Weight Capacity: Motor range to provide 17.7 to 443 in-lbf (2 to 50 Nm) lifting capacity options. [As required to operate roller shades indicated]

      - Digital Communications Network: Balanced multi-point digital actuators integrated into [stand-alone system] [building management system].

        - Basis-of-Design Product: Somfy Systems; Somfy Digital Network™ (SDN) RS485 [and animeo® IP].

        - Linear Bus Wiring: ANSI/TIA/EIA-568-B (RS-485) standard, capacitance controlled, balanced twisted pair cable.

        - Electrical Characteristics: Bus-connected control devices such as keypads, sensors and receivers powered directly from bus line using a 24V dc class 2 power supply.

        - Wire and Connectors: Category 5 balanced twisted pair cable, ethernet patch cables, RJ45 connectors and splitters.

      - Solar Management System: Digital network system integrating solar tracking, timed events, intelligent motorized window coverings, digital keypads, and weather sensors.

        - Basis-of-Design Product: Somfy Systems; animeo® IP

        - Maximum Networked Motors: 200 maximum.

        - Interface Control(s): [Wall-mounted or table top wireless keypad and receiver] [Handheld wireless keypad] [Wall-mounted digital keypad] [Personal computer] [Intelligent bus distribution network device] [Smart tablet] [and] [Smart phone] [Third-party lighting system integration] [Third-party AV system integration] <Insert control type(s)>.

      - Microprocessor Control: Electronic programmable means for setting, changing, and adjusting control features; isolated from voltage spikes and surges.

        - Basis-of-Design Product: Somfy Systems; animeo® IP.

      - Digital Keypad: Digital button-operated wall station.

        - Basis-of-Design Product: Somfy Systems; [DecoFlex digital keypad for animeo® IP] [DecoFlex digital keypad for SDN] [SDN DecoFlex digital keypad for group control].

        - Electrical Characteristics: Cable and connector to SDN bus power.

          - Power Input: 24 V dc low voltage.

          - Power Consumption: 10 mA supplied by SDN Bus.

          - Wire and Connectors: Category 5e balanced twisted pair cable, Ethernet patch cables, RJ45 connectors, and splitters.

            - Wiring Range: Up to 200 ft. (20 m) from an SDN device port.

        - Control Functions: Up, Down, Stop, [three presets] [four presets] [five presets] [ECO-MODE] [groups].

          - Buttons: [Six] [Eight]-button digital transmitter.

        - Face Plate: Lexan 945U material in standard Decora-style with color to match switch.

          - Colors: [Black] [Ivory] [White].

        - LED Indicators:

          - Red-rear of switch indicates unit is powered.

          - Red-front of switch indicates status and is flashing when transmitting.

        - Temperature Range: Ambient temperature; indoor use only.

      - General wiring specifications

        - Only mechanical connectors shall be required for data connections. (no power                           required for connector functionality)

        - Cabling configurations may be loop, spur, star, daisy chained or any combination thereof, as best suits the installation.  All cabling shall be routed to minimize the total cable length in the project.

        - Cabling configurations may be loop, spur, star, daisy chained or any combination thereof, as best suits the installation.  All cabling shall be routed to minimize the total cable length in the project.

        - Cabling shall be terminated (RJ14 / 6P6C terminations required) tested and validated prior to closing ceilings and dry walling over the cabling.

        - All data cabling must be routed to cross at right angles to and shall be at least 6” clear from any AC or DC high amperage conductors, lighting fixtures, ballasts, transformers or other low voltage or line voltage electronic devices.  Cabling shall never be subjected to a bend or crimp less than 5 times the diameter of the cable.  No anchors, zip toes or crimps shall be used to secure the cabling in manner that would compress the outer sheathing greater than its factory produced dimension.

        - Shade manufacturer to provide project level data wiring plans prior to any data cabling rough in. (requires final motor and switch locations be defined and provided).

Delete 2.10 below if Zipscreen motorized roller shades are not used on project.

- Zipscreen Motorized Roller Shade Components

    - Control System: AUTOMATE™ MOTOR SYSTEM. Shade Motor and Control System – All line and low voltage wiring rough in and termination by others.

Choose 1, 2, 3, or 4 below, deleting the one not used.

General descriptions of each Motor and Control system are included with each option below.

      - Radio Technology, 12V DC QUIET motor with built-in re-chargeable lithium ion battery and radio transceiver.

Suitable for shades up to 14’x14’

        - Tubular motor concealed inside each shade roller tube.  No line voltage, Low Voltage or communication wiring to motor location required.  Motor Operating sound level <44dB.  Plug in charger fully charges motor in 6 hours.  Motor operates >300 cycles on fully charged battery.  Charger may be permanently installed, allowing battery to serve as an emergency backup feature in a power outage.  Motor provides position reporting and “move to” position control via 2-way radio commands when operating from Mobile Device APP or connected to Automation systems.

        - Charging Options

          - Plug in 12v Charger.  1 per motor-OR-1 for every 10 motors-OR-1 per tenant space-OR-per room

          - Solar Panel Trickle charger. 1 per motor-OR- per motor (Y-Harness required)

        - Control Options

Choose any combination of control options required, from the list below.

          - Single channel wireless wall switch for radio motor control -White.

          - Two channel wireless wall switch for radio motor control -White.

          - 15 channel wireless LCD wall switch for radio motor control -White.

          - Single channel wireless handheld transmitter with concealed magnetic wall mounting clip -White.

          - 15-Channel wireless handheld LCD transmitter with concealed magnetic wall mounting clip and “Levelling Control” feature, allowing shade(s) to be operated to 10 aligned positions -White.

          - Single Channel Dry Contact Interface – Allows for full control of radio motor shade via dry contact interface. (external 12v or 24v power source required).

          - Wire-less Sun Sensor – Allows operation of one group of radio motors per sensor. 4 sensitivity level settings.

          - Wi-Fi / Serial Automation Interface.  Allows for full control, with feedback, of radio motor shade via RS485 serial data connection.  Supports Mobile Device Control through iOS or Android APPs via Wi-Fi connection.

      - Radio Technology, 12V DC QUIET motor with built-in radio transceiver.

Suitable for shades up to 14’x14’

        - Tubular motor concealed inside each shade roller tube.  No line voltage or communication wiring to motor location required.  Motor Operating sound level <43dB.  Motor provides position reporting and “move to” position control via 2-way radio commands when operating from Mobile Device APP or connected to Automation systems.

        - Power Options - Plug in 12v DC power supply 1 per motor

        - Control Options

Choose any combination of control options required, from the list below.

          - Single channel wireless wall switch for radio motor control -White.

          - Two channel wireless wall switch for radio motor control -White.

          - 15 channel wireless LCD wall switch for radio motor control -White.

          - Single channel wireless handheld transmitter with concealed magnetic wall mounting clip -White.

          - 15-Channel wireless handheld LCD transmitter with concealed magnetic wall mounting clip and “Levelling Control” feature, allowing shade(s) to be operated to 10 aligned positions -White.

          - Single Channel Dry Contact Interface – Allows for full control of radio motor shade via dry contact interface. (external 12v or 24v power source required).

          - Wire-less Sun Sensor – Allows operation of one group of radio motors per sensor. 4 sensitivity level settings.

          - Wi-Fi / Serial Automation Interface.  Allows for full control, with feedback, of radio motor shade via RS485 serial data connection.  Supports Mobile Device Control through iOS or Android APPs via Wi-Fi connection.

      - Radio Technology, 110-120V AC QUIET motor with built-in radio transceiver.

Suitable for shades up to 14’x14’

        - Tubular motor concealed inside each shade roller tube.  No communication wiring to motor location required.  Motor Operating sound level <42dB.  Motor provides position reporting and “move to” position control via 2-way radio commands when operating from Mobile Device APP or connected to Automation systems.

        - Power Options – 10-120 VAC line voltage, .85 amps or 1.0 amps (dependent on motor size)

        - Control Options

Choose any combination of control options required, from the list below.

          - Single channel wireless wall switch for radio motor control -White.

          - Two channel wireless wall switch for radio motor control -White.

          - 15 channel wireless LCD wall switch for radio motor control -White.

          - Single channel wireless handheld transmitter with concealed magnetic wall mounting clip -White.

          - 15-Channel wireless handheld LCD transmitter with concealed magnetic wall mounting clip and “Levelling Control” feature, allowing shade(s) to be operated to 10 aligned positions -White.

          - Single Channel Dry Contact Interface – Allows for full control of radio motor shade via dry contact interface. (external 12v or 24v power source required).

          - Wire-less Sun Sensor – Allows operation of one group of radio motors per sensor. 4 sensitivity level settings.

          - Wi-Fi / Serial Automation Interface.  Allows for full control, with feedback, of radio motor shade via RS485 serial data connection.  Supports Mobile Device Control through iOS or Android APPs via Wi-Fi connection.

      - Mechanical Limit, 110-120V AC QUIET motor.

Suitable for shades up to 14’x14’

        - Tubular motor concealed inside each shade roller tube.  Line voltage to motor location required.  Motor Operating sound level <44Db.

        - Power Options – 110-120 VAC line voltage, < 1.0 amps (dependent on motor size)

        - Control Options - External switches and/or controls required.  Supplied by others.  Wiring guide: White – Neutral, Black – Direction 1, Red – Direction 2, Green – Ground

- ACCESSORIES

Choose either A, B, A & C, or none below, deleting the ones not used.

Delete A, B, and C if only stationary panels, Zipscreens, or skylight shades are used on project.

    - Encloses:

      - Fascia:

        - “L”-shaped Snap-On aluminum extrusion. Square profile option only

          - 4”

        - Continuous removable extruded aluminum fascia that attaches to shade mounting brackets without the use of adhesives, magnetic strips, or exposed fasteners.

        - Fascia shall be able to be installed across two (2) or more shade bands in one piece.

        - Fascia shall fully conceal brackets, shade roller and fabric on the tube.

        - Provide bracket / fascia end caps where mounting conditions expose outside of roller shade brackets.

        - Notching of Fascia for manual chain shall not be acceptable.

      - Pocket:

        - Three-sided aluminum extrusion to conceal brackets, roller tube, fabric, and operating system above ceiling line or for recessed installation.

          - 5” x 5” actual

        - For recessed mounting in acoustical tile, or drywall ceilings as indicated on the Drawings.

        - Provide either extruded aluminum and or formed steel shade pocket, sized to accommodate roller shades, with exposed extruded aluminum closure mount, tile support and removable closure panel to provide access to shades.

      - Pocket Clip:

        - “L” shaped pocket clip for edge conditions as an integral part to the existing gyp system with an overlap of 3/4". Providing an overlap condition at the border for acoustical ceiling systems.

        - “I” shaped pocket clip for trim conditions as an integral part to the existing gyp system with an overlap of 1/16".

      - Closure panel for pockets and pocket clips:

        - Closure panel 2-11/16” in length

    - Other Accessories:

      - Room Darkening Side and / or Sill Channels:

        - Extruded aluminum with polybond edge seals and SnapLoc-mounting brackets and with concealed fastening used to eliminate light infiltration (or leakage) at side and/or bottom clearances to jambs and/or sills. Exposed fasting is not acceptable. Channels shall accept one-piece exposed blackout hembar with vinyl seal to assure side light control and sill light control.

        - Color:  Selected from manufacturer's standard colors.

      - Wire Guided Mount:

        - Top and bottom anchors for 1.2mm steel wire guide cables. Must have tension adjustment and integral wire guides in hem bar end caps. Must be corrosion resistant zinc plated, or powder coated.

Delete D below unless Zipscreens are used on project

    - Enclosure: [Full Fascia] [Semi Fascia]

      - Square front aluminum extrusion, if required, to conceal brackets, roller tube, fabric, and operating system. Bracket end covers available for exposed end brackets. Optional top and back cover provided if Full Fascia enclosure is selected.

        - Finish color as selected by architect from manufacturer’s full range of exterior grade Anodized mill-finish or Black, Paperbark, White, Gray or Bronze powder coat painted finish.

Delete 2.12 below if only manual shades are used on project.

- SOURCE QUALITY CONTROL

    - Electrical Components Testing:  Per NFPA Article 100 listed and labeled by UL, ETL or other testing agency acceptable to AHJ, marked for intended use, and tested as a system.

- -  EXECUTION

- PROJECT CONDITION

    - Environmental Requirements per SECTION 017000, and as follows:

      - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results.  Do not install products under environmental conditions outside manufacturer's absolute limits.

      - Install window treatments after finish Work including painting is complete and ambient temperature and humidity conditions are maintained at the levels indicated for Project when occupied for its intended use

    - Field Measurements: Per SECTION 017000.

- EXAMINATION

    - Examination per SECTION 017000, and as follows: Carefully examine installation areas with Installer present, for compliance with requirements affecting Work performance.

      - Verification of Conditions:  Verify that field measurements, surfaces, substrates, structural support, utility connections, tolerances, levelness, plumbness, humidity, moisture content level, cleanliness and other conditions are as required by the manufacturer, and ready to receive Work.

        - Verify each roller shade assembly will fit within its corresponding window shade pocket.

      - Proceed with installation only after unsatisfactory conditions have been corrected.

- PREPARATION

    - Preparation per SECTION 017000, and as follows:

      - Coordinate requirements for structural blocking support, and construction of properly sized shade pockets to ensure proper installation of window shade systems.

      - Coordinate installation of recessed shade pockets with ceiling construction.

      - Where applicable coordinate interface and installation of window shades with provisions of HVAC system, lighting system, fire suppression, motorized projection screens, motorized projector lift, or video projector.

      - Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.  Clean surfaces thoroughly prior to installation.

- INSTALLATION

    - Installation per SECTION 017000 and as follows:

      - Comply with shade manufacturer's written installation instructions and approved submittals.

      - Install window shades at locations indicated on Drawings and approved Shop Drawings and Window Shade Schedule.

        - Window treatment shall be installed plumb, square, level and within allowable tolerances.

        - Allow proper clearances for window hardware and operation.

      - Shade Pockets: Install shade pockets prior to or in conjunction with installation of suspended ceiling system. Attach to supporting structure with screws through top of pocket at 24 inch minimum centers.

        - Install securely, plumb, level, and in alignment with pockets.

        - After interior construction is complete, install shade and operating mechanism in pocket.

      - Install closure panels to conceal roller and operating mechanism. Do not use exposed fasteners.

      - Adjust location, level, and balance of window treatments to operate properly, smoothly, easily, safely, and free from binding or malfunction throughout entire operational range.

      - Install safety devices and tags as called for in ANSI/WCMA A100.1.

      - Roller shade installer/dealer must coordinate with project electricians prior to wiring and confirm that electrician understands wiring diagram, power requirements, and attachment details.

- FIELD QUALITY CONTROL

    - Correction of the Work per SECTION 017000, and as follows:

      - Remove, Repair and Reinstall or Restore in Place damaged items.

        - Finish touch-up damaged surface finishes.

      - Replace damaged materials or items with New if repair not acceptable to Architect.

- CLEANING AND PROTECTION

    - Clean shade assemblies and protect from damage during construction operations.

      - Remove temporary coverings and protection of adjacent work areas.

      - If damage occurs, remove and replace damaged components or entire unit as required to provide units in their original, undamaged condition.

      - Touch-up any areas damaged during roller shade installation.

      - Clean up debris that resulted from shade installation

- CLOSEOUT ACTIVITIES

    - Substantial Completion Requirements per SECTION 017000, and as follows:

      - Provide Final Cleaning immediately prior to Substantial Completion inspection.

        - Protection: Remove manufacturer’s and other installed protection immediately prior to Substantial Completion inspection, unless manufacturer requires otherwise.

      - Perform required Demonstration and Training with Owner’s designated staff.

        - Engage Installer to train Owner's maintenance personnel to adjust, operate and maintain roller shade systems.

END OF SECTION